Network Fundamentals Lecture ６ Notes

Communications Media

- Communications media for LANs may be either by means of wiring or by wireless approaches

- Wiring approaches consist of:

- -Twisted-pair cabling

- -Coaxial cable media

- -Fibre-optic cabling

- Wireless approaches consist of:

- -Radio or infrared transmission

- -Satellites as a special case of radio broadcast

The Wired Approach

- Modern wiring plans normally follow standard structured cabling methods

- -Wired closets on each floor of a building

- -An orderly cabling installation

- Utilizing cabling racks

- -Patch panel

- -Wiring distribution

- -Network access devices

Cabling Media Choices

- The network developer has a number of alternative cabling media choices

- -Twisted pair, coaxial cable and fibre optic cabling

- The cabling selection issues include;

- -The required data rate (including growth considerations)

- -The level of electrical interference (Would your AM radio pick up a lot of interference in this environment?)

- -The maximum cabling length that must be supported

- -Cost

Unshielded Twisted-Pair Cables

- The least expensive media (unshielded)

- Capable of handling up to 100m

- Unshielded twisted pair (UTP)

- -Data capacity grades defined by EIA/TIA 568 (ISO 11801)

- -Category 3 (to 10 Mbit/s or more)

- -Category 4 (to 20 Mbit/s or more)

- -Category 5 (to 100 Mbit/s or more)

- Category 5e and 6 are used extensively today (to 1000Mbit/s and above)

RJ-45 Connector	Unshielded Twisted Pair (UTP) 8 Cables (4 Pairs)

Multiplexing

- Combining a number of different data/telephone channels

- On the same cable-copper and fibre

Category 6 Cabling

- The latest form of UTP cabling is category 6

- It adds additional quality assurances beyond category 5e

- It is more likely to work as planned after you’ve paid to have it installed

- -Labour costs is by far the largest cost component in cabling

- It comes in 2 forms

- -UTP and screened Twisted Pair (ScTP)

- -ScTP has a layer of metallic foil to improve its interference rejection

- May utilize larger-diameter copper wires

- -Helps for ‘power over Ethernet’ situations

Shielded Twisted-Pair

- Shielded twisted pair (STP)

- -Primary used by IBM

- -Should be better than UTP – Shields help prevent interface from outside signal, also help prevent interference to outside signals

- Token ring environments may include a mix of UTP and STP cabling

Coaxial Cable – Legacy

- Low noise (low bit-error rate)

- Used in a variety of networking applications

- -In IBM network (e.g. cluster controllers)

- -In earlier Ethernet (10 Mbit/s)

- The shielding may include multiple layers of foil and/or braid as shown below

- -This is a Thickwire Ethernet cable

Fibre-Optic Cables

- Extremely high data rates

- -More than 100 mbit/s for LAN uses

- -More than 10 times that for telephone company links

- Usage is typically with two unidirectional links, with one fibre in each direction

- Converts electrical signals to light and back to electrical

- Very small size

- -Hair-like fibre-optical strand (125- micron outer diameter)

- -Light-conducting core size may be 62.5 microns (Called 62.5/125 – micron fibe

- -May use 50/125 – micron fibre (Europe)

Fibre-Optic Characteristics

- LAN usage is usually multimode, graded index

- -Multimode supports different light transmission paths (pulses spread out in time)

- -Graded index resists pulse spreading due to different transmission speeds

- Use single-mode fibre for very high speed and/or long distance

- Best available communications medium

- -Excellent electrical noise immunity

- -Difficult to tap (security)

- -Lightweight

- -Small size

Fibre-Optic Issues

- A single fibre may support multiple light beams

- -Dense wave division multiplexing (DWDM)

- -Up to 100 or more simultaneous transmissions

- -Only used with single-mode fibre

- Media converters are available between different media types

Wireless Communications

- There are several different forms of wireless communications

- Infrared beams (point-to-point)

- Point-to-point microwave

- -Requires ‘line of sight’ between antennas

- -Antennas are often mounted on towers

- -Requires a license

- Cellular telephones

- -Used for voice and data

- -E-mail access and some Web access

- -‘Always on’ possibilities